# MaRS Recruitment Task #0 Exploration Task

# Rules and Regulations:-

1. You are advised to familiarize yourself with each subsystem and give a brief about your 	understanding of them, consult any website for resources or AI tools but develop your own 	unique understatement. Below we gave a brief description of all the subsystems you can use 	as a reference.

2. Give a detailed description of any one subsystem of your choice that fascinates you, 	describe its functionality, its interactions with other subsystems, etc. Rest other 		subsystems’ description need not be as detailed.

3. Consider the potential problems that might arise in the subsystem of your choice and 	propose solutions for them.

4. We encourage you to understand each subsystem, its functionality and the type of work 	being done on the rover in relation to it. Any unique ideas if you guys have in order to 	achieve the 	requirements are very much appreciated.

### 5. You can include the workflow within and between subsystems(How each subsystem 	is communicating with others to meet the task requirements. Tip - You can show a 	         	flowchart ).

6. Your designs and information presentation skills are valued and subject to additional marks.

7. This documentation can be done in any form like word document,presentation, physical 	submission or any other means.

8. You are expected to understand the given subsystems and apply their knowledge 		creatively by documenting a rover of their own imagination. The task is not just 		theoretical you should conceptualize how each subsystem works together in 			your own rover design and document it clearly.

# Requirements:-

1.It's recommended to make your own rover sketch or design and specify each and every subsystem of the rover in your sketch or design .

2.Your rover should be able to traverse through Martian terrain - Uneven terrain with rocks and boulders in the path.

# Pro-Tip:-

Try to explain the rover and its subsystem in your own language (don’t just completely rely on AI tools) because you also have to develop approaches for your three chosen tasks. We are pretty sure that in order to complete chosen tasks, your knowledge about each and every subsystem is necessary.

# Subsystems:-

## Operating System:-

An Operating System (OS) is software that manages all the hardware resources of a computer, facilitating communication between software applications and hardware components.You're most likely using a Windows or Mac machine right now, both are different in visual design and how they operate. Just like Windows and MacOS, Linux is just another in a list of many operating systems and one of the most popular in the world powering smart cars, android devices, supercomputers, home appliances, enterprise servers, and more. For robotics, Linux Ubuntu is preferred because it is open-source, stable, and supports many robotic tools. ROS (Robot Operating System) provides libraries and tools to help software developers create robot applications. ROS is an open-source, meta-operating system for our rover.

Ref. - https://wiki.ros.org/ROS/Introduction

## Computing Device:-

A rover requires a computing device capable of handling real-time data processing, sensor fusion, and autonomous decision-making. Computing devices in robotics are categorized into microprocessors, embedded computers, and general-purpose computers. Microcontrollers such as the ESP32, and STM32 are compact, energy-efficient units used for sensor control and peripheral management. Embedded computers like Jetson AGX Orin and Raspberry Pi etc are designed for providing GPU acceleration for tasks like image processing and neural network computations. General-purpose computing devices, such as laptops and desktops, are primarily used for simulation, software development, and debugging. The Jetson AGX Orin is used as the main computing device in our rover due to its high-performance capabilities, enabling

real-time navigation, sensor data processing and complex algorithms processing.

Ref.- https://developer.nvidia.com/embedded/learn/jetson-agx-orin-devkit-user-guide/index.html Bonus - Get to know about the technical specification of the device you are using.

## Sensors:-

Sensors enable the rover to perceive and interact with its surroundings by gathering environmental data. Some of the sensors We use - Visual sensors, such as the ZED 2i Stereo Camera and standard webcams, provide depth perception and object recognition capabilities, allowing the rover to navigate effectively. Environmental sensors like the DHT11 measure temperature and humidity, helping the rover adapt to changing conditions. Other critical sensors include IMUs (Inertial Measurement Units) for motion tracking and orientation, LIDAR (Light Detection and Ranging) for high-precision 3D mapping and obstacle detection, ultrasonic sensors for short-range obstacle avoidance, and GPS (Global Positioning System) for outdoor navigation and localization. By integrating multiple sensors, the rover can develop a comprehensive understanding of its environment, enabling smooth autonomous operation.

Ref. - https://www.stereolabs.com/docs

## Autonomous Navigation:-

### The basic necessities of an autonomous navigation system are to avoid obstacles and collisions, stay within the designated operational area and be predictable, that is to avoid sudden erratic movements. The system thus perceives the environment and commands the rover to achieve goals.

The autonomous navigation system of the rover consists of perception of the system’s environment, storing the perceived data in a convenient form, localisation using this data, decision making from the perceived data and execution of the decisions. How the system perceives and what it perceives, how the perceived data is utilized to make decisions, what forms of decision made are relevant to navigation and what questions does it answer to make these decisions are what composes an autonomous system.

What makes a relevant goal for an autonomous navigation system, how will your system behave in specific assumed conditions, what are the minimal rules that your system would follow, etc., are a few things you have to consider along with the tools and concepts you will use to achieve these, while developing an autonomous navigation system.

## Communication system:-

### A communication system in a rover is a critical component that allows the rover to send and receive information from its operators and the outside world.

Communication systems in rovers are typically designed to operate in harsh environments and may need to be able to withstand extreme temperatures, radiation, and other environmental factors that can interfere with communication signals. Engineers must design communication systems that are reliable, efficient, and can transmit large amounts of data quickly and accurately to ensure that rovers can successfully carry out their missions.

## Control system:-

### A control system in a rover is responsible for controlling and coordinating the various subsystems in the rover, enabling it to perform its intended mission.Sensors are critical components that enable the control system to sense and measure the rover's environment, including parameters such as temperature, pressure, humidity, and terrain features.

The control system in a rover must also be able to adapt to unexpected situations and respond to changing mission objectives. Overall, It is the backbone of the rover where it takes care of internal wireline communication between subsystems and based on the tele operation of the rover, it handles the information and performs the transmission and completion of task accordingly.

## Power electronics:-

### A power electronic system in a rover is responsible for managing the electrical power supply and distribution for the various systems and subsystems in the rover.

Ensuring proper power supply is a crucial aspect of a rover's functionality, as even if all subsystems are operational, insufficient power can hinder their performance or even cause damage. Also consider built-in safety measures to ensure safe operation.

## Manipulator:-

### A manipulator in itself is a complex system that brings together various mechanical, electronics, and software components to achieve a specific task. In essence, a manipulator is a robotic arm that is designed to perform precise movements and actions, such as lifting and manipulating objects, cutting, drilling,etc.

The autonomous manipulation system has several key features like being aware of its state and the surrounding workspace, ability to achieve the goal state from the current state, recognising the subjects and objects to interact with to complete a task, etc. Consider the build configuration(like DoF, number of joints, types of joints, number of links) so as to aid the manipulator to interpret a given goal. How it will use the sensory inputs, how it will make decisions and plans and how it will execute them. Also consider the tools and concepts you will employ to achieve these.

## Chassis:-

### The chassis of the Mars rover is the mechanical structure that provides the framework and support for all the rover's components, including the scientific instruments, power systems, and wheels. It also houses the rover's mobility system.

Consider the choice of materials for the chassis fabrication and its role in protecting critical components. Develop strategies to mitigate risks associated with components during operation, including protection against damage.

## Drivetrain:-

### The rover is a remarkable vehicle designed to explore and navigate challenging terrain. To achieve its purpose, it requires a robust suspension system and reliable wheels that can withstand and navigate the rough terrain environment. The suspension system provides smooth maneuvering of the rover keeping all the wheels in contact with the ground, minimizing the impact of rough terrain on the rover and its instruments.

It is recommended to look into the different types of suspension system and their advantages and how they may impact the rover. Choose your suspension system wisely! The wheels also play an important role in providing the required traction to climb elevations and obstacles.

Understand the failure nodes or uncertainties that may arise that can cause hindrance in the mission.

## Electronics Safety System:-

### The rover's electronics safety system, essential for shielding sensitive components in space, also guards against electronic failures like reverse polarity. By incorporating these features, the system enhances reliability and minimizes the risk of disruptions caused by electronic malfunctions.

These safety measures extend to protection during rover movement.Mention possible errors and their solutions, including backup systems for such failures.Thus, ensuring mission functionality and reliability in extraterrestrial environments.

## Life Detection System :-

### The main aim of the rover is to identify the existence of life on Mars and hence the requirement of a Life Detection system that can collect, store different samples and identify any possibility of existing or existing life.

Since the rover is an autonomous vehicle the cache must be automated and function without any intervention. You are requested to look into ways of sample collection and the different types of chemical analysis and mechanisms that automate these processes.

## Science Cache:-

### A Science Cache is a collection of scientific equipment and sensors to conduct various experiments and studies .These caches are designed to analyze soil composition, atmospheric conditions, and other geological features.

It is recommended to create your own cache system while thinking about microbial life detection, predictions that can be made with the surroundings of mars.

Reference Video - https://youtu.be/MFyv8mtRPCA?si=Ke1L6BJk65KKsJhh

## Geology Of Mars:-

### It is the study of rocks, minerals, and landforms of the planet to learn about its past and present conditions. It is used to analyze geology features of mars for choosing the potential landing site.

- Research on different types of rocks and minerals. (mainly based on the crater and volcanic eruption areas also consider the chances of microbial life around the area) on the mars surface.

- Choose a potential landing site on mars surface.

( https://murray-lab.caltech.edu/CTX/V01/SceneView/MurrayLabCTXmosaic.html ) Give precise and brief reasoning of the two above mentioned points.

# Presentation:-

Prepare a poster of your rover, indicating its specification and speciality. (Reference Poster)

# Resources:-

Rover Challenges:

- International Rover Challenge (IRC)	- https://roverchallenge.org/irc/

- International Rover Design Challenge (IRDC) - https://roverchallenge.org/irdc/

- University Rover Challenge (URC)	- http://urc.marssociety.org/home

- European Rover Challenge (ERC)	- https://roverchallenge.eu/en/main-page/

- About ERC	-  https://www.youtube.com/watch?v=C98zKFM3Vf4

(For a better understanding try to read the guidelines & requirements of respective challenges.) (Note: IRC, International Rover Challenge formerly known as Indian Rover Challenge)

Rover Challenges Guidelines:

- ERC - https://roverchallenge.eu/en/competitor-zone/

  - https://drive.google.com/drive/folders/1jBBQ20hDFdNI3CTzUWVbCXaszJs9SeEG

  - https://drive.google.com/drive/folders/1riF3epdDdEECZQXffx1hBp-GKXPlb49U

- URC  - https://drive.google.com/file/d/1LY5VX9Psww8Bx4ub_AWMgUlajPZQ2X2A/view

System Acceptance Review (SAR) Videos Link:

1. URC SAR -            https://youtu.be/Ji6JOx3XZDQ?si=_OYsZqIM7W5posaS

URC SAR 2024:

WVU Team Mountaineers System Acceptance Review (SAR) for 2024 Unive… Michigan Mars Rover Team SAR 2023

Michigan Mars Rover Team SAR 2022

URC 2021 - Systems Acceptance Review - Hindsight AIUB ROBOTIC CREW SAR URC 2020

Monash Nova Rover Team | 2019 University Rover Challenge SAR

- URC SAR 2023-

- URC SAR 2022 -

- URC SAR 2021 -

- URC SAR 2020 -

- URC SAR 2019 -

7.International Rover Challenge (IRC) SAR

Technocrats, VIT Chennai - (2019) https://www.youtube.com/watch?v=XryJQQ79oyw

- (2020) https://www.youtube.com/watch?v=rzEgMwaUgXU UDAYAVANI, Manipal - https://www.youtube.com/watch?v=2X9tBh82yAs

DRISHTI, NIT Surat	- https://www.youtube.com/watch?v=EFN9qkRg_rM

8.Other mixed playlists *** 		 https://youtube.com/playlist?list=PLePCRhtF7zrmb8goTujSUVENIoxLkd3Hb https://www.youtube.com/playlist?list=PLn0LlFXnT7ssQFc6Usa0RjvJLmmIrcw_l https://www.youtube.com/playlist?list=PLLc1NzV8QgKbMxaZFt6qtqzhiMiNNhDO5 https://www.youtube.com/playlist?list=PLXL3uCh2oMp6WqojsNYWsrcX7I9KBr7C7

MaRS IIITDM Youtube Channel -	Team Shunya | ERC 2022 Remote Edition | Video Material

BTS during Finals Submission | Team SHUNYA | European Rover Challenge 2023 Onsite Edition